Smart Factory Vision Monitor

Technical Analysis Document

Introduction to Computer Vision    EPICODE Institute of Technology

Author: Stefano Bertaccini  |  Exam deadline: July 2026

# 1. Problem Statement

## 1.1 Context and Motivation

Visual quality control is one of those manufacturing steps that is easy to underestimate until something goes wrong. In most small and medium factories, a human operator checks each part as it moves to the next stage, a process that works well in low-volume settings but breaks down as production speed increases. Fatigue, distraction, and subjective judgment mean that defect rates under manual inspection are rarely consistent.

The idea behind this project is straightforward: replace (or, more realistically, assist) the human inspector with a camera placed above the workstation. Every part that passes under the lens is automatically classified as good or defective. If the classification is reliable enough, the system can flag anomalies in real time and stop the production line before a batch of defective parts reaches assembly.

Why computer vision rather than a simple sensor? Sensors measure physical quantities such as weight, temperature, and dimensions. Defects like surface scratches, color anomalies, or structural deformations are inherently visual and spatial: they only become apparent when you look at the part. CV is a natural fit.

## 1.2 Problem Definition

Given a single image of a manufactured part taken by a fixed overhead camera, the system must accomplish two things:

- Classify the part as good or defective (binary classification).

- Verify that the part is positioned within the expected workstation zone. Parts that fall outside the region of interest are flagged regardless of their apparent quality, because a mispositioned part suggests a handling error that the classifier was not trained to handle.

## 1.3 Why This Problem Is Relevant

Manufacturing defects have an outsized downstream cost. McKinsey (2022) estimated that quality failures cost the global manufacturing industry around $300 billion per year; most of it is attributable not to the defective parts themselves but to late-stage detection that triggers recalls and rework. Early detection at the workstation level, before parts enter assembly, is the most cost-effective intervention point.

From a technical standpoint, the problem is also interesting because it sits at the intersection of classical and deep learning CV: the dataset is too small to train large models from scratch, which forces deliberate choices about transfer learning and feature engineering.

## 1.4 Scope and Limitations

This project covers binary classification on static images. Real-time video streaming, 3D reconstruction, and multi-part inspection are explicitly out of scope, not because they are unimportant, but because addressing them well would require a different experimental setup and more data. The dataset used is the MVTec Anomaly Detection benchmark (Bergmann et al., 2021), which serves as a proxy for real factory imagery. The primary evaluation target is the metal_nut category, with secondary evaluation on six texture categories where the classical model is expected to struggle.

# 2. Methodology

## 2.1 Pipeline Architecture

The system is built as four independent, sequentially connected modules. Keeping them separate was a deliberate design choice: it makes each stage testable in isolation and makes it straightforward to swap out, say, the backbone without touching preprocessing.

| Stage | Module | Responsibility |
| --- | --- | --- |
| 1 | src/preprocessing.py | Load → RGB → resize 224×224 → normalize [0,1] → augment |
| 2 | src/features.py | HOG feature extraction (classical path) |
| 3 | src/models/ | classical.py: SVM+HOG ¦ deep.py: EfficientNet-B0 fine-tuned |
| 4 | src/postprocessing.py + evaluate.py | ROI zone check, NMS, morphological ops, metrics |

## 2.2 Stage 1  Preprocessing

Every image goes through the same preprocessing pipeline regardless of which model will process it downstream. The key design decisions are summarized below.

| Decision | Choice | Rationale |
| --- | --- | --- |
| Target resolution | 224 × 224 px | Standard ImageNet input size, no architectural changes needed for transfer learning |
| Color space | RGB | Matches torch vision convention; avoids silent BGR→RGB bugs that are hard to catch during training. |
| Normalization range | [0, 1] | Shared between classical and DL pipelines. ImageNet means/std are applied separately in the Data Loader, so preprocessing stays model-agnostic |
| Noise reduction | Gaussian blur k=3 | MVTec images show visible sensor noise. k=3 removes it without blurring the texture boundaries that HOG relies on |
| Augmentation | Horizontal flip only | Metal nuts are left-right symmetric but have a defined orientation (they can't be flipped upside down without creating an impossible part), so only a horizontal flip was enabled. |

## 2.3 Stage 2  Feature Engineering

Two feature extraction strategies are implemented in parallel, one classical and one learned, to allow a direct performance comparison under identical preprocessing and evaluation conditions.

### Histogram of Oriented Gradients (HOG)

HOG describes the local distribution of edge directions in small spatial cells. For industrial surfaces, edges and gradient changes are the most consistent indicators of structural defects such as dents, bends, and scratches, which is why HOG was chosen as the classical descriptor rather than simpler alternatives like LBP or raw pixel intensity.

Parameter choices: 9 orientation bins (40° resolution per bin), 8×8 pixels per cell (a reasonable trade-off between spatial precision and descriptor length on 224×224 images), 2×2 cells per block for L2 block normalization. These are the standard HOG settings established by Dalal and Triggs (2005) and serve as a good starting point.

### EfficientNet-B0 Backbone (learned features)

For the deep learning path, feature extraction is handled by an EfficientNet-B0 backbone pretrained on ImageNet. The backbone outputs 1280-dimensional feature vectors, which are fed into a custom two-layer classification head: Linear(1280→256) → ReLU → Dropout(0.4) → Linear(256→2).

Why EfficientNet-B0 and not ResNet50? EfficientNet-B0 has 5.3M parameters versus ResNet50’s 25M. On a dataset with roughly 200–400 training samples per category, fewer parameters significantly reduce the risk of overfitting. Additionally, EfficientNet’s compound scaling tends to produce richer texture representations, which matters more here than object-level semantics.

## 2.4 Stage 3  Core Models

### Classical baseline: SVM on HOG features

The HOG descriptor is flattened to a 1D vector and fed into a Support Vector Machine with RBF kernel (C=1.0) after StandardScaler normalization. The RBF kernel implicitly maps the HOG vector into a higher-dimensional space where a linear separation becomes possible. This is the standard approach for texture classification tasks and provides a solid, well-understood, lower bound for the deep model.

### EfficientNet-B0 with two-phase fine-tuning

Training proceeds in two phases. In Phase 1 (10 epochs, lr=1e-3), the backbone is frozen, and only the custom head is trained. This prevents the catastrophic forgetting of ImageNet features that would occur if the full network were fine-tuned from a randomly initialized head. In Phase 2 (10 epochs, lr=1e-5), the entire network is unfrozen and trained at a deliberately low learning rate to adapt the backbone to industrial texture statistics while preserving the pretrained representations.

A second training run (V2) addressed the class imbalance problem discovered during evaluation: with a 2.6:1 good/defective ratio, unweighted CrossEntropyLoss biased the model toward predicting “good”. The fix was a one-line change: CrossEntropyLoss(weight=[1.0, 2.60]), with Phase 2 extended to 20 epochs. This improved F1 from 0.634 to 0.792 and reduced false negatives from 15 to 9 on the metal_nut test set.

## 2.5 Stage 4  Post-processing and ROI Zone Check

The ROI zone check runs after classification and acts as a spatial safety gate. The idea is that a part positioned outside the expected workstation zone produces feature distributions the classifier was never trained on, so it makes sense to reject it regardless of the defect score.

The ROI is defined as the central 60% of the image (20% margin on each side). Part localization uses background thresholding with a threshold of 30/255. MVTec images have a near-black background, so the part stands out as a bright foreground blob. The bounding box of the largest contour is computed and compared against the ROI; if the overlap falls below 80%, the part is flagged as out of position.

Known limitation: MVTec images are close-up shots where the part fills most of the frame. The 20% margin was designed for wide-field factory cameras. For MVTec evaluation, a 5% margin (ROI-margin 0.05) is necessary. In a real deployment, the margin would be calibrated to the specific camera’s field of view.

# 3. Experimental Results

## 3.1 Dataset Statistics

The primary evaluation is performed on the metal_nut category. Both models use the same stratified 70/30 split (random_state=42) to ensure directly comparable results.

| Split | Good | Defective | Total |
| --- | --- | --- | --- |
| Combined (train+test) | 242 | 93 | 335 |
| Train (70%, stratified) | ~169 | ~65 | ~234 |
| Test (30%, stratified) | ~73 | ~28 | ~101 |

Defect types in the test set: bent, color, flip, scratch. Note that this is a supervised setup; defect examples are included in training. This differs from MVTec’s intended unsupervised benchmark (train on good images only), which is worth acknowledging: our F1 scores are higher than those reported in the unsupervised literature precisely because the classifier has seen defect examples during training.

## 3.2 Classification Results  metal_nut

Recall is treated as the primary metric throughout because, in industrial inspection, a missed defect (false negative) carries a higher downstream cost than a false alarm (false positive).

| Model | Accuracy | F1 (defect) | Recall (defect) | Precision (defect) | FN / 28 |
| --- | --- | --- | --- | --- | --- |
| HOG + SVM | 83.2% | 0.605 | 46.4% | 86.7% | 15 |
| EfficientNet V1 (t=0.5) | 85.1% | 0.634 | 46.4% | 100.0% | 15 |
| EfficientNet V2 (t=0.5) ★ | 88.1% | 0.792 | 67.9% | 95.0% | 9 |

The most relevant comparison is V1 vs V2: both use the same architecture, but V2 introduces class-weighted loss. The effect is substantial, false negatives drop from 15 to 9 (a 40% reduction) with only a 5 pp precision cost. Threshold tuning on V1 (t=0.3) had produced a similar recall improvement, but V2 achieved it without sacrificing precision at the default threshold.

## 3.3 Threshold Sensitivity  EfficientNet-B0 V1

These results were produced during the V1 evaluation and motivated the weighted-loss fix applied in V2.

| Threshold | Accuracy | F1 (defect) | Recall | Precision |
| --- | --- | --- | --- | --- |
| 0.5 (default) | 85.1% | 0.634 | 46.4% | 100.0% |
| 0.4 | 88.1% | 0.727 | 57.1% | 100.0% |
| 0.3 | 89.1% | 0.784 | 71.4% | 87.0% |
| 0.2 | 84.2% | 0.727 | 85.7% | 63.2% |

## 3.4 Per-Category Evaluation  All 15 MVTec Categories

HOG+SVM was evaluated on all 15 categories following the per-category protocol of Bergmann et al. (2021): one model was trained and tested per category, with a stratified 70/30 split.

| Result | HOG+SVM | EfficientNet V2 (selected) |
| --- | --- | --- |
| Categories evaluated | 15 / 15 | 7 / 15 |
| Mean F1 (defect) | 0.263 | 0.755 (on 7 trained) |
| Mean Recall (defect) | 0.271 |  |
| Functional (F1 > 0) | 6 / 15 | 7 / 7 |

HOG fails completely (F1=0.000) on 9 categories: carpet, leather, wood, tile, grid, cable, screw, transistor, hazelnut. These are all cases where defects manifest as color shifts or subtle texture changes with minimal gradient variation, exactly the information HOG discards by design. EfficientNet V2, trained in the six categories where HOG scored zero plus metal_nut, achieved strong results (leather 0.839, wood 0.824, carpet 0.821, metal_nut 0.792, cable 0.667, tile 0.649) with the notable exception of grid (0.111), where the regular repeating pattern confuses the binary classifier.

## 3.5 Grad-CAM Explainability Analysis

Grad-CAM (Selvaraju et al., 2017) was applied to EfficientNet-B0 V2 to identify which image regions drive the classification decision. The target layer is the backbone. Features[-1][0]  the final convolutional layer before global average pooling.

| Defect type | Defect probability | Verdict | Deletion test (mask top-20% heatmap) |
| --- | --- | --- | --- |
| Good (train) | 6.8% | PASS |  |
| Good (test) | 19.7% | PASS |  |
| bent | 67.2% | REJECT | Prob increases after masking the structural/global feature |
| scratch | 50.3% | REJECT | Prob: 50.3% → 0.0% (causally linked region confirmed) |
| color | 66.7% | REJECT | Prob: 66.7% → 0.0% (causally linked region confirmed) |
| flip | 99.7% | REJECT |  |

Pointing accuracy, the fraction of defective images where the heatmap maximum falls inside the ground truth defect mask was 4.3% (4/93). This is low but expected: a binary classifier trained on image-level labels learns global features rather than local defect localization. The deletion test is more informative: for scratch and color defects, masking the attended region drops the defect probability to zero, confirming that the model is responding to a real visual signal rather than noise. The bent case is different; the defect affects the part's overall shape, so no single local region dominates the decision.

For pixel-precise defect localization, a dedicated anomaly-detection approach, such as PatchCore, would be needed. That is identified as future work.

# 4. Failure Analysis

## 4.1 HOG+SVM Failure Modes

| Failure | Root cause | Evidence | Possible mitigation |
| --- | --- | --- | --- |
| False negatives on color/texture defects | HOG encodes gradient direction, not color. A color-shift anomaly produces no gradient change, so the HOG vector of a defective part is nearly identical to a good one. | F1 = 0.000 on 9/15 texture categories | Use learned features (EfficientNet) for color/texture defect types |
| False negatives on fine scratches | HOG cell size is 8×8 px. A scratch narrower than a single cell is averaged over neighboring cells and effectively disappears. | screw category: thread damage undetected | Reduce pixels_per_cell to (4,4) doubles vector length, increases compute |
| Orientation sensitivity | HOG is partially but not fully rotation-invariant. The same part, photographed at a slightly different angle, can yield a meaningfully different descriptor. | flip category: orientation alone drives the decision | Add rotation augmentation during training |

## 4.2 EfficientNet Failure Modes

| Failure | Root cause | Evidence | Possible mitigation |
| --- | --- | --- | --- |
| Class-imbalance bias (V1) | 2.6:1 good/defective ratio with unweighted CrossEntropyLoss biases the model toward predicting the majority class. | V1 Recall = 46.4%, Precision = 100% at default threshold | Class-weighted loss implemented in V2, F1 improved from 0.634 to 0.792 |
| Grid category poor performance | The regular repeating grid pattern makes it hard to distinguish a missing thread from a correctly spaced one using image-level labels only. | F1 = 0.111 on the grid category | Anomaly localization approach (PatchCore) is better suited for periodic textures. |
| No localization output | The model outputs a single class probability with no spatial information about where the defect is. | Grad-CAM pointing accuracy 4.3% | Segmentation decoder or unsupervised anomaly map (e.g., PatchCore) |

## 4.3 ROI Zone Check Limitation

The 20% margin was calibrated for wide-field cameras, in which the workstation occupies roughly 60% of the frame. MVTec images are close-up shots in which the part occupies over 90% of the frame, so the default ROI check would reject most images as out of position. The ROI-margin 0.05 flag is required for MVTec evaluation. This is not a problem in practice in a real installation; the margin would be configured once during camera setup, but it is worth noting as a gap between the demo environment and a production deployment.

# 5. Ethical Considerations

## 5.1 Dataset Bias and Generalizability

The models are trained exclusively on metal_nut (and six other categories for the texture evaluation). Deploying the metal_nut classifier on a different part geometry or material without retraining would produce unreliable results, potentially worse than random, because the model would apply a learned decision boundary that has no relationship to the new part’s visual properties. There is currently no mechanism in the system to detect this scenario and warn the operator.

A related concern is lighting bias: MVTec images were captured in a controlled photographic setup. A real factory camera under varying lighting would shift the distribution of normalized pixel values, and the model’s behavior under that shift is untested.

## 5.2 Worker Privacy

If cameras are installed above workstations, workers may appear in captured frames, especially during part loading and unloading. The ROI zone check provides a partial mitigation by cropping attention to the part zone, but it does not prevent the full frame from being stored. Any production deployment would need to address this through explicit informed consent, data minimization (process frames in real time without long-term storage), and image anonymization if workers are consistently in frame.

## 5.3 Automation and Labor Impact

Automated inspection systems reduce the need for manual quality control operators. This is not inherently problematic, but it should be communicated transparently to all stakeholders, including workers, before deployment. The system is better positioned as a tool that assists human inspectors rather than replaces them, at least until its error rates are thoroughly characterized under real production conditions.

## 5.4 False Negative Risk in Safety-Critical Applications

A false negative, a defective part classified as good, has direct safety implications if the component is used in load-bearing or safety-critical assemblies. The current false negative rate of ~32% (9/28 defective test parts under EfficientNet V2) means that roughly one in three defective parts would pass inspection undetected. This is a significant improvement over the HOG baseline (54% FN rate), but it is still too high for safety-critical applications without additional human review. The system should be understood as a first-pass filter, not a final gate.

# References

Bergmann, P., Batzner, K., Fauser, M., Sattlegger, D., & Steger, C. (2021). The MVTec Anomaly Detection Dataset: A Comprehensive Real-World Dataset for Unsupervised Anomaly Detection. International Journal of Computer Vision, 129(4), 1038–1059.

Dalal, N., & Triggs, B. (2005). Histograms of Oriented Gradients for Human Detection. Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 886–893.

Selvaraju, R. R., Cogswell, M., Das, A., Vedantam, R., Parikh, D., & Batra, D. (2017). Grad-CAM: Visual Explanations from Deep Networks via Gradient-Based Localization. Proceedings of the IEEE International Conference on Computer Vision (ICCV), 618–626.

Tan, M., & Le, Q. V. (2019). EfficientNet: Rethinking Model Scaling for Convolutional Neural Networks. Proceedings of the International Conference on Machine Learning (ICML), 6105–6114.

McKinsey & Company. (2022). Capturing the true value of Industry 4.0. McKinsey Global Institute.